--- In Revcon_Curious@yahoogroups.com, "bobwoodruff" <woodruf1@p...>

wrote:

Here's some info I put together years ago on my previous '75 Revcon.

No gauartees they are still in business, or even make the parts

anymore.  Most of the NAPA numbers have changed to their new inventory

system, and I'm not sure even they know the cross-references.  But

hey, if any of them work, it's better than nothing.  Right?

DISCLAIMER: None of the information below is to be taken as a

recommendation.  It is information only, and to be used according to

your own judgement and experience.

This compilation deals with finding parts and replacement part numbers

for Revcons.  My main interest in creating this manual was to condense

the valuable information into a central location.  Hopefully, to be

used by others like me, who are out in the "ozone" trying in vain to

find parts.  This information may be out-of-date, however, it might

give you a starting point to find the parts you need. Unless otherwise

stated, most information applies to the "flat face" Revcon Motorcoach.

AXLE FLANGE BOLTS ï?? GM Repair Manuals call for the replacement of the

axle 12 point flange bolts when the axles are removed.  If you are a

person who likes to do it by the book you might want to keep a

replacement number on hand. They too can be ordered from Cinnabar,

Part number 394777 at 75 cents each. It will insure their staying there when properly torqued

THE AXLE FLANGE BOLT NUMBER IS A GOOD NUMBER THE 75 CENT NUMBER IS AMUSING THE LAST I BOUGHT WERE 3 PLUS DOLLARS EACH I GOT MINE THROUGH MY LOCAL GM DEALER ,THE WASHERS ARE NO LAONGER AVAILABLE HAVE TO LOOK THEM UP AS BEST OR CLOSE AS YOU ARE ABLE

AXLE, REAR SUSPENSION CENTER BUSHINGS - Some members are finding that

the center bushings for the rear axle suspension are contributing to

the uneven wear of the tires.

BAD NEWS/GOOD NEWS - As some of you have probably discovered GM no

longer supplies exhaust manifolds for the Olds 455 Engine. Aftermarket

headers are being made to fit in the early Revcon. Erick's RV and

Performance Center, 275 S. 7th Ave. Sequim, WA 98382 Phone (206)

683-3696 is now furnishing headers that fit your coach.  GOT AN EMAIL FROM ERIC AND HE IS STILL MAKING

AND SELLING HEADERS

Eric says his headers are built with heavy flanges  to with stand rust and corrosion

Eric Davis
Eric's RV Performance Center
275 S. 7th Ave.
Sequim, WA. 98382

eric@ericsrv.com
800 488 3697

BALL JOINTS (LOWER) - Moog Part No. K6215 is correct for '76 and '77

models but '74 uses the regular Tornado part. These joints are not the

same as those for the Tornado automobile.  They may be obtained from

automotive suppliers using the GM Part No. 9755841 or Moog K-6215. One

nut may require grinding for clearance.

COULD NOT GET THE GM # TO CROSS REFerence  BUT THE MOOG IS A GOOD NUMBER THE PART ABOUT IT NOT BEING A STOCK TORONADO PART IS NOT RIGHT HOWEVER, IT IS! THE MOOG PART IS JUST A HEAVIER CASTING THAN THE NAPA OEM STUFF. AND THE TAPER IS THICKER THE EARLIER UNITS WILL HAVE TO BE REAMED TO MALE THE MOOG WORK BUT WOULD SEEM TO BE WORTH WHILE UPGRADE.

Moog  K6215

NAPA    # NCP 2601392              $109.00  as of  2/4/2009

BATTERIES (DEEP CYCLE VS STARTING) - Trojan batteries specialize in

deep cycle batteries. 1-800-423-6569, or 1-800-246-2550 in Georgia.

BELL CRANK ï?? Part no. Torrington MR-18 or McGill MR-18.

BRAKES (FRONT) CHEV 454 ENGINES - The pad number is Wagner 7054 and

the rotor is Raybestos 6084. Rotors need to have 1/8th inch machined

from the circumference. Calipers are Wagner CR-102430 for the right

and Wagner CR-102431 for the left.  Overhaul kits are Bendix 66713.

Remote Solenoid Switch: Delco 1115616

Napa 8T71

Echlin 1259

BRAKE CYLINDERS (REAR WHEELS) - Flat noses earlier than '77 are

Raybestos and NAPA Nos. 14521 Left and 14522 Right.

77 REAR WHEEL CYLINDERS ARE

RAYBESTOS- WC37218 LEFT

WC37219 RIGHT

THESE ARE WHEEL CYLINDERS FROM LATE 70 TO EARLY 90 FORD 1 TON SINGLE TIRE TWO WHEEL DRIVE PICKUPS

BRAKE DRUMS - Raybestos Rear Brake Drum, Part Number 1350 can be

modified to fit the flat nose.  Machine the center hole to the correct

diameter and then drill new lug-bolt holes to match.

BRAKE DRUMS ï?? Try Marty Moore in San Diego, California.

BRAKE PADS (FRONT) - Redesigned brake pads increase braking on the

front wheels. Bendix D152 or Wagner MX7084R.

BRAKE PARTS (REAR) ï?? Wheel cylinders NAPA 1421 left and 14522 right.

Brake shoes Bendix 358, same as NAPA RS358R.

Raybestos              NAPA

Kit-R.H. H2577              80847

L.H. H2576              80846

R.H. H1541              80586 Star Adjustment Only

L.H. H1540              80585		"		"

Kit contains springs, adjustment cable, star adjuster and lever.

BRAKE MASTER CYLINDER (Flat Nose) - NAPA Number 36437.      GOOD NUMBER  $704 yikes!!!!

BRAKE MASTER CYLINDER (SLANT NOSE) - Bendix 222952IMP.

BRAKE ROTORS ï?? Slant nose Raybestos number is 6084 & NAPA 85731 (both

require machining.  Flat nose coaches NAPA 85518.

BRAKE SHOES ï?? Flat nose Bendix 358 NAPA AE 358R.  Slant nose 314.

77 FLAT NOSE TAKES THE AE 314R SHOES

AT LEAST MINE DOES

DIPSTICK TUBE REMOVAL (ENGINE OIL) - No doubt many of you have had the

dipstick tube break near the point where it is inserted in the block.

If you haven't and you have enough miles on your coach, you will have

the problem of removing the portion, which is still in the block and

hopefully, enough of it sticking out above the insertion point to use

in its removal.  The first step using any of the suggested methods of

removal requires the removal of the left exhaust manifold.  A member

found that, after removing the manifold and turning himself into a

pretzel, he was able to grasp the short protruding piece of the

remaining tube with a large pair of diagonals and carefully work the

tube out of the block.  Another suggestion by a member of the GMC

Chapter is to thread a metric tap of appropriate size into the

remaining portion of the tube and after grasping it with vise grips,

use a small hammer to lift it out.  If you don't mind putting out a

few bucks to make the job easier and possibly get by without removing

the exhaust manifold, a tool, Part Number DSP-109, can be obtained

from Mondello Performance Products, Van Nuys, CA, (818) 994-6910.  The

cost is approximately $35.

ENGINE CHANGE (455 TORONADO) - The Revcon's do not have the same cam

specifications the Toronado had. Be sure that it has the proper cam.

Cam Kits are available for the Olds 455 for use in motorhomes.  The

kits include valve springs, retainers, etc.  New hydraulic lifters

must be installed. Part numbers Wolverine WG 1112, Engle JM 1618, and

Crower 56-260.

NOT REAL SURE I AGREE WITH THIS STATEMENT HARD TO ARGUE WITH A CAM THAT HAS ALL ITS TORQUE AT 2000 RPM, ALL THE ABOVE CAMS ARE SO CLOSE TO THE STOCK SPECS THAT I DON'T THINK IT WOULD MAKE ANY DIFFERENCE ESPECIALLY IF THE TORO CAM WAS GOOD.

ENGINE COOLANT ï?? Don't use Sierra Brand as an engine coolant. See GMC

Service Bulletin 436201.

I LIKE TO USE A WATER WETTER ALSO LIKE REDLINE OR OTHERS HELPS WHEN THE SYSTEM IS CLEAN

EXHAUST MANIFOLDS - Olds 455 and 403 engines. Cinnabar Engineering

Inc. 116 Orval Street, Sandusky, Michigan 48471. Phone 1-800-720-2227.

EXHAUST MANIFOLD BOLTS ï?? GM 9440954, grade 9 cap screw or GM 9420462.

Use anti-seize lubricant.

EXHAUST MANIFOLD EMERGENCY REPAIRS - Aremco Products Pyro Putty works

well. Dries in 30 minutes and hardens by engine heat. Caspro Company

(440-423-0809)

FAN CLUTCH - Chevrolet part Number 15643265 or Delco 15-4445.

FINAL DRIVE RING AND PINION GEAR SETS (SLANT NOSE) - Slant nose

Revcons were delivered with a 3.73 Ratio, a 4.10 final drive and

pinion set is available, GM 15594112.

FINAL DRIVE SEAL (RIGHT HAND) - Victor 47464 and Chicago 14994. Left

hand are GM 40440, Victor 47465, and National 9185.

LOWER CONTROL ARM - Cinnabar Engineering at 1-800-720-2227 or Globe

Motor Corp., Orlando, Florida, at 1-800-872-1462.

LOWER CONTROL ARM BUSHINGS (SLANT NOSE) - Henderson Lineup Brake and

RV Service, in Grants Pass, Oregon, 1-800-245-8309.

LUG NUTS AND STUDS - NAPA.

Nuts- Left Hand 641-2007          Studs- Left Hand 641-2525

Right Hand 641-2006                Right Hand 641-2524

OIL DIPSTICK TUBE - GM Part No. 702000.

PARTS (Slant Nose)-

Front End

Brake Caliper RH Napa 3424055M (w/pads).  Brake Caliper LH Napa

3424054M (w/pads). Brake Caliper Repair Kit Napa 702. Brake Rotors

Napa #85731 (machine 0.25 in. from outside diameter). Master Cylinder

Napa 39622 or GM 1 4009146. Brake Hose Wagner F42184. Upper Ball

Joints (L & R) Napa 260-1079. Knuckle Seals Napa 49727. Wheel Bearings

Napa 387A. Shock Absorbers Bilstein F-4B46-1319.

Rear End

Brake Shoes Napa V314R. Wheel Cylinders Left Napa 4637132, Right Napa

4637133. Hub Seal (Oil filled) Napa 62446. Inner Hub Bearing Napa

25580. Inner Hub Bearing Race Napa 25520. Outer Hub Bearing Napa 2475.

Outer Hub Bearing Race Napa 2420. Brake Drums Dexter #8-187A. Brake

Hoses                    Napa 36695 and 31669.

POWER BRAKE BOOSTER ï?? A Midland Power Brake is available thru NAPA as

Part Number C4035H.

POWER STEERING PUMP (Flat Nose) - GM 7818023. Auto Line 557068, or

Fenco SP 16044. Filter is AC PF883.

NOT SURE ONTHESE NUMBERS COULD NOT GET THEM TO CHECK OUTGOOD ONES BELOW

NAPA -  NSP817434   REMAN PUMP AND RESERVOIR     62.49  7.20 CORE

NAPA-   NSP811391   REMAN PUMP                                    49.99  5.60 CORE

NAPA     NSP81FLT2 AND FLT3 MAG INLINE FILTERS        23 AND 26 DOLLARS

SHIFT CABLE FOR SLANT NOSE - Davis Power Hydraulics, Dallas Texas.

Tel 214-637-1311.

SHOCK ABSORBERS ï?? Rear Bilstein B46-0248 or Gabriel 601000. Front

Bilstein F4-B46-0940-HI.

SHOCKS ABSORBERS (SLANT NOSE) - Bilstein #F4-B46-1319-WO. Requires

modifications. A 1/2" x 5/8" x 1 1/2' metal sleeve for the lower

connection. Upper connection drilled out to 5/8' dia w/new bolts.

Universal Joints ï??Spicer 5-1205X.

VENT CAPS - VIKING in Port St. Richey, FL. (Cap only)

WHEELS - 1973-1979 coaches. Made to order.  Standard Wheel and Rim,

P.O. Box 1715, Harrisburg, PA 17105.  Tel.717-238-6777.

WHEEL BEARINGS (FRONT)- Cinnabar Part No. 123516771

NOT SURE ON THE CINNABAR NUMBER BUT I USED THE TORONADO BEARINGS BECAUSE THAT IS WHAT MINE HAD JUST CROSSED THE NUMBERS AT THE LOCAL BEARING HOUSE

WHEEL BEARING ASSEMBLY, FRONT (IMPROVED) - Front wheel bearing

assembly using three bearings and a fitting for greasing the bearing

without removing the hub and bearings.  Wallace Bearing and Hub

Co.9004 Nogales Hwy.  Tucson, AZ 65706, Phone (602) 294-3076.

WINDSHIELDS - Pittsburgh Automotive Glass dealers. REV 002 for right

and REV 001 for left. And Marty Moore.

REVCON PARTS & REPAIR SERVICES

Caspro Company

Novelty, Ohio 44072-0390

216-423-0809

Chuck Stoddard or Cynthia

Claseo.Manufacturing Corp.

5603 Commerce Dr.

Orlando, FL 32839

1-800-275-4462

Jim Bounda

Conley Travel Center

800 Cortez Road West

Bradenton, FL 34207

1-800-777-8531

D & D Custom Service & Manufacturing

29391 U.S. Hwy 33 Went

Elkhart, IN 46516

1-800-551-9149   219-294-6852

Chuck Mitsehelen

DCM (Windshield Supplier)

Elkhart, IN

Talk to Sue

219-294-6989

Emerson Power Transmission Corp.

7120 Buffington

P.O. BOX 728

Florence, KY 41042

Morse Chain (For Transfer Case)

Gateway Motorhome Co.

4897 Schumacher Rd

High Ridge, MO 63049

1-800-654-0374

Glass Parts

1560 North East Marx

Portland, Oregon

503-254-9694

Doug or Dianne Prentiss

Golby Motor Corp

5611 Commerce Drive

Orlando, FL 32839

Jim Debes

Lares Corp

13517 HWY 65

Ham Lake, MN 55304

Tel 1-800-334-5749

Transfer Flow (Gas tanks)

916-893-5209

(Custom made)

Marty Moore RV Services

4019 Hicock St,

San Diego, CA 92110

619-299-3720

Patterson Orillia

21 Matehedash St. South

Ortilla, Ont. L3V6H9

705-325-1322

R.B. (Dick) Paterson, Pres

Todd Engineering (Converter)

Tucson, AZ

602-889-9333

RV & Custom Coach

1217 N. Parker St.

Orange, CA 92667

714-771-4998

714-532-4205

Vic Morse

Specialty Tires

412-349-9010

Osbornes RV Service

6001 Holly NE

Alburguerue, NM 87113

Ed or Blake Osborne

SOME REPLACEMENT PART NUMBERS FOR EARLY REVCONS

Belt, Alternator 403 & 455 Engine

GMC405600

Dsyco 15590

Napa 25-14483

Gates 7570

Belt For Water Pump & PS Engines

Gates (403 & 455 Engine) 7450

Belt Air Conditioner

Gates (403 Engine) 7603

Gates (455 Engine) 7619

Brakes, Front Hose

ELS SP-5363

Napa (Slightly Smaller) 36678

Wagner F-86578

Brakes, Front, Caliper Assembly LH

ELS 6139

Napa (Rebuilt) 42-2010

Wagner (Rebuilt) CR-80940-L

Brakes, Front, Caliper Assembly RH

ELS 6140

Napa (Rebuilt) 42-2010

Wagner (Rebuilt) CR-80941-R

Brakes, Front, Disc Pads

Bendix D-52

Tel 717-238-6777

Wagner (Heavy Duty) MX-7084R

Brakes, Master Cylinder

Napa 36437

Brakes, Rear, Shoes

Bendix 358

ELS ES-358R

Brakes, Rear Assembly

Raybestos Kit RH H2577

Raybestos Kit LH H2576

Raybestos Star Bolt RH 41541

Napa Kit RH 80847

Napa Kit LH 80846

Napa Star Bolt RH 80586

Napa Star Bolt LH 80585

Brakes, Rear Wheel Cylinders

Napa 14521 RH (1977)

Napa 14522 LH (1977)

Engine, Carburetor

(Out of production)

Holley 2-200 (With Modification)

Rebuild Part Numbers Vary With

Carb. Numbers

Engine Timing Chains

(Varies w/ engine No.)

Engine Water Pump

GM 231887 (Before HEI)

GM 231887 (With HEI)

TRW FP-1454 (With HEI)

Final Drive Seal LH

National 9185

Victor 47465

Final Drive Seal RH

Chicago 14994

Victor 47464

Final Drive Seal

(Plsles Gear)

GM 1371622

Front Axle CV Jts

See J.C. Whitney

Catalog

Front Axle CV Boots

Moog CV-2456

Front Axle CV Jt Clamp

Moog CV 3485 (Outer)

Moog CV 3427 (Inner)

Front Suspension Ball Jt

Moog K5238 (Upper)

Moog K6215 (Lower)

Front Suspension Torsion Bars

Marty Moore RV Service

San Diego, Ca

619-299-3720

Front Suspension Cntrl Arm Bush

Moog K7104 (Upper)

Moog K5222 (Lower)

Napa 2673299 (Lower)

Front Suspension Tie Rod Ends

Moog ES-361-R (Inner)

Moog ES 412-RL (Outer)

Front Wheel Bearing Seals

Napa 47471 (Inner)

Napa 47470 (Outer)

National 5123 (Inner)

National 5109 (Outer)

Rear Wheel Bearings

Bower 3877 (Inner)

Bower 3820 (Inner)

Bower 25877 (Outer)

Bower 25821 (Outer)

Real Wheel Seal

National 415349

Wheel Lugs

Napa 641-2525 LH

Napa 641-2006 RH

Wheel Nuts

Napa 641-2007 LH

Napa 641-2006 RH

Wiper Blades

Napa 60-2008

Wheels

Standard Wheel and Rim

Harrisberg, PA

Tel 717-238-6777

-or-

Conley Travel Center

Bradenton, Fl 34207

Tel 813-755-8531

Front Parking Light Hldr

International 443002C91

Steering Gear Box Rebuild

Caspro Company

Novelty, OH 216-423-0809

-or-

Lares Corporation

Ham Lake, MN

1-800-334-5749

-or-

Holinder H954 (`69-`79

Chev Stepvan)

--- End forwarded message ---